Superscalar-LEGv8 · Episode 3
Rename Stage — Structures, Algorithms & RTL Realities

§1 — The Rename Stage as a System

The rename stage transforms architectural registers into physical registers while preserving correctness, throughput, and recoverability.

Core operations per cycle:
- Allocate physical registers
- Map architectural to physical
- Forward intra-cycle dependencies

§2 — Core Structures

Rename Table:
- map_table (speculative)
- committed_table (architectural state)

Free List:
- 64-bit mask tracking free physical registers
- Round-robin allocation pointer

Implicit Structures:
- temp_mask and search_ptr used for intra-cycle consistency

X0 Handling:
- Always mapped to p0, never allocated

§3 — Algorithms

Allocation (Round-Robin):
- Start from alloc_ptr
- Scan for free register
- Update temp_mask to prevent duplicates

Multi-Port Allocation:
- Sequential semantics using temp_mask

Forwarding:
- Later lanes override map_table using earlier lane allocations

Commit Interaction:
- committed_table updated on commit
- Old physical registers freed

Flush Recovery:
- map_table restored from committed_table

§4 — Pipeline Behavior

Backpressure:
- Should depend on alloc_valid signals

Hazards:
- Partial allocation must stall or compact lanes

§5 — SystemVerilog Notes

Key RTL insights:
- always_comb loops create priority logic
- automatic variables prevent latches
- modulo operations optimized for power-of-2
- multi-dimensional arrays flatten in synthesis
- sequential loops imply priority, not parallel writes

§6 — Key Strengths

✔ Multi-port rename correctness
✔ Intra-cycle forwarding
✔ Clean speculative vs committed separation
✔ Fair allocation via round-robin

§7 — Summary

The rename stage is a transactional system that assigns unique physical registers to eliminate false dependencies while enabling rollback and high throughput.